Lab 4

Analog (continuous-time) filters: design and analysis systems

EECS3451

Name: Chethana Wickramasinghe

Student Number: 214866511

Professor: Peter Lian

18th of March	 2021

1. Introduction:

Using MATLAB to design analog filters such as Butterworth, Chebyshev (Type I and II), and Elliptic filters and to learn how to apply filter to real-world problem. This report is mainly answering the provided questions using MATLAB. Results will demonstrate the use of MATLAB properly in analysing and designing Analog filters.

2. Equipment: MATLAB

3. Results and discussion:

The answers to provided question are as follows,

# Laboratory Exercise 1

## Q1

Butterworth filter

Wp =10;

Ws =20;

Pb = 0.9; %Passband

Sb = 0.15; %Stopband

Rp = -20*log10(Pb);

Rs = -20*log10(Sb);

[N,Wc]= buttord(Wp,Ws,Rp,Rs, 's')

[num,den]=butter(N,Wc,'s')

Ht=tf(num,den)

[H,w]=freqs(num,den);

% plot magnitude response

plot(w, abs(H));

xline(Wp,'--k',sprintf('Wp=%0.2f', Wp))

xline(Ws,'--k',sprintf('Ws=%0.2f', Ws))

yline(Pb,'--k',sprintf('delta_p=%0.2f', Pb))

yline(Sb,'--k',sprintf('delta_s=%0.2f', Sb))

xlabel('Frequency-w');

ylabel('Magnitude (dB)');

title('Magnitude Repsonse (dB)');

- Transfer function = 2.428e04 s4+ 32.62 s3+ 532 s2+ 5082 s + 2.428e04

- Magnitude response plot

Bode plot

- Verify

Type II Chebyshev Filter

Wp =10;

Ws =20;

Pb = 0.9; %Passband

Sb = 0.15; %Stopband

Rp = -20*log10(Pb);

Rs = -20*log10(Sb);

[N,Wn]= cheb2ord(Wp,Ws,Rp,Rs, 's')

[num,den]=cheby2(N,Rs,Wn,'s')

Ht=tf(num,den)

[H,w]=freqs(num,den);

% plot magnitude response

plot(w, abs(H));

xline(Wp,'--k',sprintf('Wp=%0.2f', Wp))

xline(Ws,'--k',sprintf('Ws=%0.2f', Ws))

yline(Pb,'--k',sprintf('delta_p=%0.2f', Pb))

yline(Sb,'--k',sprintf('delta_s=%0.2f', Sb))

xlabel('Frequency-w');

ylabel('Magnitude (dB)');

title('Magnitude Repsonse (dB)');

% Bode plot

bode(Ht);

- Transfer function = 7.599 s2 + 2825 s3 + 26.75 s2 + 328.9 s + 2825

- Magnitude response plot

Bode plot

- Verify

Difference between Butterworth and Type II Chebyshev Filters

Butterworth filter is flat during pass band and stop band, while in Type II Chebyshev it is flat during pass band but has ripples during stop band.

Transition band of Butterworth filter is larger than Type II Chebyshev.

## Q2

Type I Chebyshev filter

Wp =15;

Ws =20;

Pb = 0.9; %Passband

Sb = 0.15; %Stopband

Rp = -20*log10(Pb);

Rs = -20*log10(Sb);

[N,Wn]= cheb1ord(Wp,Ws,Rp,Rs, 's');

[num,den]=cheby1(N,Rp,Wn,'s');

Ht=tf(num,den)

[H,w]=freqs(num,den);

% plot magnitude response

plot(w, abs(H));

xlabel('Frequency-w');

ylabel('Magnitude (dB)');

title('Magnitude Repsonse (dB)');

xline(Wp,'--k',sprintf('Wp=%0.2f', Wp))

xline(Ws,'--k',sprintf('Ws=%0.2f', Ws))

yline(Pb,'--k',sprintf('delta_p=%0.2f', Pb))

yline(Sb,'--k',sprintf('delta_s=%0.2f', Sb))

% Bode plot

bode(Ht);

- Transfer function =  9.799e04s5 + 14.5 s4 + 386.4 s3 + 3422 s2 + 3.035e04 s + 9.799e04

- Magnitude response plot

Bode plot

- Verify

Elliptic Filter

Wp =10;

Ws =20;

Pb = 0.9; %Passband

Sb = 0.15; %Stopband

Rp = -20*log10(Pb);

Rs = -20*log10(Sb);

[N,Wn]= cheb2ord(Wp,Ws,Rp,Rs, 's')

[num,den]=cheby2(N,Rs,Wn,'s')

Ht=tf(num,den)

[H,w]=freqs(num,den);

% plot magnitude response

plot(w, abs(H));

xline(Wp,'--k',sprintf('Wp=%0.2f', Wp))

xline(Ws,'--k',sprintf('Ws=%0.2f', Ws))

yline(Pb,'--k',sprintf('delta_p=%0.2f', Pb))

yline(Sb,'--k',sprintf('delta_s=%0.2f', Sb))

xlabel('Frequency-w');

ylabel('Magnitude (dB)');

title('Magnitude Repsonse (dB)');

% Bode plot

bode(Ht);

- Transfer function = 6.412 s2 + 2538 s3 + 15.08 s2 + 282.8 s + 2538

- Magnitude response plot

Bode plot

- Verify

Difference between Type I Chebyshev Filters and Elliptic Filter

Type I Chebyshev filter is flat during stop band and pass band ripples, while in Elliptic it has ripples in both pass band and stop band.

Transition band of Type I Chebyshev filter is larger than Elliptic.

# Laboratory Exercise 2

This Exercise is done using Elliptic and Butterworth filters

% 1) Using Elliptic filter

% high pass filter specifications

wp=1; ws=0.1; Rp=-20*log(0.9); Rs=20;

[N,Hwn] = ellipord(wp,ws,Rp,Rs,'s') ;

[num4,den4] = ellip(N,Rp,Rs,Hwn,'high','s') ;

H4 = tf(num4,den4);

Transfer function= 0.7846 s2- 8.711e-17 s + 0.1155s2+ 0.8387 s + 1.155

[H,w]=freqs(num4,den4);

plot(w, abs(H));

xlabel('Frequency-w');

ylabel('Magnitude (dB)');

title('High pass');

bode(H4);

% low pass filter specifications

wp=10; ws=60; Rp=-20*log(0.9); Rs=40;

[N,Lwn] = ellipord(wp,ws,Rp,Rs,'s') ;

[num4,den4] = ellip(N,Rp,Rs,Lwn,'s') ;

H4 = tf(num4,den4);

Transfer function=0.01 s2+ 63.77s2+ 7.778 s + 81.28

[H,w]=freqs(num4,den4);

plot(w, abs(H));

xlabel('Frequency-w');

ylabel('Magnitude (dB)');

title('Low pass');

bode(H4)

% Band pass filter specifications

[num,den] = ellip(N,Rp,Rs,[Hwn Lwn],'s') ;

H = tf(num,den);

Transfer function=0.01 s4- 1.776e-17 s3+ 51.85 s2- 1.776e-16 s + 1s4+ 7 s3+ 85.83 s2+ 70 s + 100

[PH,w]=freqs(num,den);

plot(w, abs(PH));

xlabel('Frequency-w');

ylabel('Magnitude (dB)');

title('Band pass');

% Verification

xline(1,'--k','Wp1=1')

xline(10,'--k','Wp2=10')

xline(0.1,'--k','Ws1=0.1')

xline(60,'--k','Ws2=60')

yline(0.9,'--k','Pb=0.9')

yline(0.1,'--k','Sb1=0.1')

yline(0.01,'--k','Sb2=0.01')

closeup of a small range

bode(H)

% 2) Using Butterworth filter

% high pass filter specifications

wp=1; ws=0.1; Rp=-20*log(0.9); Rs=20;

[N, Hwc] = buttord(wp,ws,Rp,Rs,'s');

[num1,den1] = butter(N,Hwc,'high','s');

H4 = tf(num1,den1);

Transfer function=s2s2+ 0.4461 s + 0.0995

[H,w]=freqs(num1,den1);

plot(w, abs(H));

xlabel('Frequency-w');

ylabel('Magnitude (dB)');

title('High pass');

bode(H4)

% low pass filter specifications

wp=10; ws=60; Rp=-20*log(0.9); Rs=40;

[N, Lwc] = buttord(wp,ws,Rp,Rs,'s');

[num1,den1] = butter(N,Lwc,'s');

H4 = tf(num1,den1);

Transfer function=2160s3+ 25.85 s2+ 334.2 s + 2160

[H,w]=freqs(num1,den1);

plot(w, abs(H));

xlabel('Frequency-w');

ylabel('Magnitude (dB)');

title('Low pass');

bode(H4)

% Band pass filter specifications

[num,den] = butter(N,[Hwc Lwc],'s') ;

H = tf(num,den);

Transfer function=2006 s3s6+ 25.22 s5+ 330.3 s4+ 2212 s3+ 1347 s2+ 419.4 s + 67.8

[PH,w]=freqs(num,den);

plot(w, abs(PH));

xlabel('Frequency-w');

ylabel('Magnitude (dB)');

title('Band pass');

% Verifications

xline(1,'--k','Wp1=1')

xline(10,'--k','Wp2=10')

xline(0.1,'--k','Ws1=0.1')

xline(60,'--k','Ws2=60')

yline(0.9,'--k','Pb=0.9')

yline(0.1,'--k','Sb1=0.1')

yline(0.01,'--k','Sb2=0.01')

bode(H)

4. Conclusion: state what you learn from this lab, lab objectives you achieved, and any difficulties you met.

Learned how to use Butterworth, Chebyshev (Type I and II), and Elliptic filters in MATLAB to design analog filters and how to build a filter that could be used in an ECG system.

All the questions were answered using MATLAB and was able to get a better understanding of high pass, low pass and band pass filters.

Building a band pass filter for an EGC system was challenging, but turns out its not that hard to make one.